Electronic Supplementary Material

HPLC-MS/MS multiclass determination of steroid hormones in environmental waters after preconcentration on the carbonaceous sorbent HA-C@silica

Francesca Merlo1, Andrea Speltini2, Federica Maraschi1, Michela Sturini1, Antonella Profumo1*

1Department of Chemistry, University of Pavia, via Taramelli 12, 27100 Pavia, Italy

2Department of Drug Sciences, University of Pavia, via Taramelli 12, 27100 Pavia, Italy

*E-mail: antonella.profumo@unipv.it

Table S1. Physical-chemical parameters of the water samples.

|  | Tap water | Staffora River | Ticino River | Tanaro River | Wastewater effluent* |
| --- | --- | --- | --- | --- | --- |
| pH | 7.7 | 7.4 | 7.6 | 7.6 | 6.5 |
| Conductivity at 20 °C (S cm-1) | 270 | 307.8 | 223 | 432 | 551 |
| Cl- (mg L-1) | 5.0 | 4.2 | 9.5 | 15.7 | 52.4 |
| NO3- (mg L-1) | 0.6 | 1.7 | 7.9 | 8.0 | 9.5 |
| SO42- (mg L-1) | 5.0 | 13.5 | 32.6 | 41.2 | 64 |
| Ca2+ (mg L-1) | 36.1 | 56 | 31.5 | 63 | 62.1 |
| Mg2+ (mg L-1) | 8.1 | 8.1 | 6.2 | 10.0 | 10.1 |
| Mn2+ (mg L-1) | < 0.05 | < 0.05 | < 0.05 | < 0.05 | < 0.05 |
| Fe (mg L-1) | 0.045 | 0.12 | 0.06 | 0.3 | 0.076 |

*other parameters (mg L-1): COD 24, SS 2, Ptot 0.45, NH4+ 3.7, NO2- 0.4.

Table S2. Mean recovery (%) in tap water (500 mL, 1000 ng L-1) at different pH extraction.

|  | Mean recovery (%) |  |
| --- | --- | --- |
|  | pH 3 | pH 7.5 |
| E1 | 86 | 104 |
| E2 | 87 | 111 |
| EE2 | 85 | 105 |
| PROG | 48 | 50 |
| H-PROG | 93 | 109 |
| M-PROG | 74 | 99 |
| TST | 81 | 102 |

RSD < 10%, n=3

Table S3. Mean recovery (%) in tap water (500 mL, 1000 ng L-1) with different elution solvents.

| Mean recovery (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MeOH | MeOH + 2% v/v NH4OH |  | MeOH + 0.2% v/v formic acid | ACN |
| E1 | 104 | 101 |  | 89 | 75 |
| E2 | 111 | 99 |  | 84 | 79 |
| EE2 | 105 | 97 |  | 85 | 83 |
| PROG | 50 | 48 |  | 49 | 83 |
| H-PROG | 109 | 97 |  | 99 | 83 |
| M-PROG | 99 | 98 |  | 99 | 86 |
| TST | 102 | 93 |  | 89 | 60 |

RSD < 10%, n=3

Table S4. Optimized MRM conditions for the HPLC-ESI-MS/MS analysis of GCs.

|  | Precursor Ion* (m/z) | MRM product ions (m/z) | Dwell time | Fragmentor Energy (V) | Collision Energy (V) | Polarity |
| --- | --- | --- | --- | --- | --- | --- |
| CORT | 361 | 163.2 | 15 | 130 | 20 | Positive |
|  |  | 105 | 15 | 130 | 48 |  |
| H-CORT | 363 | 121.1 | 15 | 124 | 20 | Positive |
|  |  | 91 | 15 | 124 | 68 |  |
| PRED | 359 | 341.3 | 15 | 76 | 4 | Positive |
|  |  | 147.1 | 15 | 76 | 24 |  |
| PREDLO | 343 | 147.1 | 15 | 112 | 20 | Positive |
|  |  | 91 | 15 | 112 | 72 |  |
| BETA | 373 | 355.3 | 15 | 82 | 8 | Positive |
|  |  | 147.1 | 15 | 82 | 24 |  |
| DEXA | 373 | 355.3 | 15 | 76 | 4 | Positive |
|  |  | 91 | 15 | 76 | 80 |  |
| TRIAM | 415 | 397.3 | 15 | 130 | 8 | Positive |
|  |  | 91 | 15 | 130 | 80 |  |
| FLUO | 495 | 475.3 | 15 | 88 | 0 | Positive |
|  |  | 337.3 | 15 | 88 | 8 |  |

*[M+H]+ adduct.

Figure S1. A MRM chromatogram of a SPE eluate obtained from pre-concentration of 1 L of Ticino River sample.

Figure S2. A MRM chromatogram of a SPE eluate obtained from pre-concentration of 1 L of Ticino River sample.