Supporting information

Preparation of Quercus mongolica leaf-derived porous carbon with a large specific surface area for highly effective removal of dye and antibiotic from water

Qinghua Zhoua,c,1, Yingying Wua,d,1, Huanjia Chena, Guanya Zhua, Yupeng Zhanga, Dadong Lianga,b,*, Guang Chenb, Shanshan Tangb,*

aCollege of Resource and Environment, Jilin Agricultural University, Changchun 130118, China

bKey Laboratory of Straw Comprehensive Utilization and Black Soil Conservation, The Ministry of Education, College of Life, Jilin Agricultural University, Changchun, Jilin 130118, China

cCollege of Resources, Sichuan Agricultural University, Chengdu 611130, China

dCollege of Plant Protection, Nanjing Agricultural University, Nanjing 210095, China

*Corresponding authors E-mail address: liangdadong@jlau.edu.cn (D.D. Liang), tangshanshan81@163.com (S.S. Tang). Tel: +086-0431-84532995.

1These authors contributed equally to this work.

Contents

1. Mechanism of chemical activation

2. Fig. S1 The relationship between the RhB adsorption capacities of porous carbon products and the activation conditions including (a) temperature, (b) the ratio of NaOH to carbon, and (c) activation time using NaOH as the activator.

3. Fig. S2 The (a) C1s and (b) O1s XPS spectra of PCQL.

4. Fig. S3 (a) the initial pH and finial pH of NaCl aqueous solutions after PCQL samples were added; (b) Effect of pH on the adsorption capacities of PCQL toward RhB.

5. Table S1 Parameters of adsorption kinetic models for RhB adsorption on PCQL.

6. Table S2 Parameters of adsorption kinetic models for TC adsorption on PCQL.

S1. Mechanism of chemical activation

When the biochar is heated under N2 atmosphere with the presence of NaOH, the possible chemical reaction steps are listed as following:

6NaOH+2C→2Na+2Na2CO3+3H2↑            (S1)

Na2CO3→Na2O+CO2↑                    (S2)

CO2+C→2CO                       (S3)

Through the reaction of biochar and NaOH, a large number of micro- and mesopores occur in the surface of biochar, and the specific surface area of biochar may largely increased (Norouzi et al., 2018).

Fig. S1 The relationship between the RhB adsorption capacities of porous carbon products and the activation conditions including (a) temperature, (b) the ratio of NaOH to carbon, and (c) activation time using NaOH as the activator.

Fig. S2 The (a) C1s and (b) O1s XPS spectra of PCQL.

Fig. S3 (a) the initial pH and finial pH of NaCl aqueous solutions after PCQL samples were added; (b) Effect of pH on the adsorption capacities of PCQL toward RhB.

Table S1 Parameters of adsorption kinetic models for RhB adsorption on PCQL.

| Models | Equation |  | Co (mg L-1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 | 200 | 300 | 400 |
| Pseudo-first-order | qt=qe(1−e−k1t) | k1 (min-1) | 0.15 | 0.14 | 0.15 | 0.19 |
|  |  | qe (mg g-1) | 867.0 | 1108.7 | 1195.3 | 1259.9 |
|  |  | R2 | 0.9925 | 0.9897 | 0.9895 | 0.9947 |
|  |  | SSE | 508.48 | 1143.68 | 1354.07 | 755.13 |
| Pseudo-second-order | qt=k2qe2t1+k2qet | k2 (g mg-1 min-1) | 3.6×10-4 | 2.4×10-4 | 2.5×10-4 | 3.8×10-4 |
|  |  | qe (mg g-1) | 907.3 | 1168.1 | 1254.2 | 1302.6 |
|  |  | R2 | 0.9998 | 0.9999 | 0.9999 | 0.9999 |
|  |  | SSE | 10.24 | 7.53 | 12.61 | 5.26 |
| Intra-particle diffusion |  | ki,1 (L g-1) | 183.92 | 230.40 | 252.33 | 278.23 |
|  |  | C | 25.05 | 28.91 | 35.38 | 45.89 |
|  |  | R2 | 0.9371 | 0.9464 | 0.9335 | 0.9092 |
|  |  | SSE | 11601.31 | 15448.38 | 23139.13 | 38921.74 |
|  |  | ki,2 (L g-1) | 17.31 | 22.68 | 33.22 | 17.84 |
|  |  | C | 733.86 | 926.31 | 941.44 | 1125.19 |
|  |  | R2 | 0.9895 | 0.9366 | 0.9299 | 0.9299 |
|  |  | SSE | 6.03 | 64.85 | 408.04 | 44.57 |
|  |  | ki,3 (L g-1) | 1.86 | 5.33 | 5.80 | 3.45 |
|  |  | C | 862.83 | 1071.89 | 1155.18 | 1240.22 |
|  |  | R2 | 0.7516 | 0.9412 | 0.9535 | 0.9641 |
|  |  | SSE | 14.54 | 24.51 | 22.71 | 6.17 |

Table S2 Parameters of adsorption kinetic models for TC adsorption on PCQL.

| Models | Equation |  | Co (mg L-1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 | 200 | 300 | 400 |
| Pseudo-first-order | qt=qe(1−e−k1t) | k1 (min-1) | 0.20 | 0.20 | 0.17 | 0.17 |
|  |  | qe (mg g-1) | 1254.4 | 1228.7 | 1162.5 | 897.4 |
|  |  | R2 | 0.9964 | 0.9968 | 0.9940 | 0.9925 |
|  |  | SSE | 496.08 | 421.73 | 723.78 | 537.67 |
| Pseudo-second-order | qt=k2qe2t1+k2qet | k2 (g mg-1 min-1) | 4.4×10-4 | 3.6×10-4 | 5.0×10-4 | 4.5×10-4 |
|  |  | qe (mg g-1) | 932.8 | 1206.5 | 1262.1 | 1290.7 |
|  |  | R2 | 0.9999 | 0.9999 | 0.9998 | 0.9998 |
|  |  | SSE | 3.6 | 6.66 | 16.26 | 20.73 |
| Intra-particle diffusion |  | ki,1 (L g-1) | 194.86 | 254.87 | 277.99 | 281.50 |
|  |  | C | 31.01 | 38.61 | 46.54 | 47.17 |
|  |  | R2 | 0.9151 | 0.9228 | 0.9065 | 0.9064 |
|  |  | SSE | 17776.59 | 27557.90 | 40030.64 | 41129.54 |
|  |  | ki,2 (L g-1) | 14.43 | 17.28 | 12.19 | 10.71 |
|  |  | C | 785.94 | 1027.19 | 1133.68 | 1170.07 |
|  |  | R2 | 0.9443 | 0.9559 | 0.9923 | 0.8115 |
|  |  | SSE | 22.96 | 25.85 | 2.17 | 47.12 |
|  |  | ki,3 (L g-1) | 2.06 | 2.11 | 2.52 | 3.99 |
|  |  | C | 891.89 | 1160.89 | 1216.66 | 1224.52 |
|  |  | R2 | 0.9515 | 0.8997 | 0.7253 | 0.9182 |
|  |  | SSE | 3.01 | 6.77 | 30.10 | 19.45 |

References

Norouzi, S., Heidari, M., Alipour, V., Rahmanian, O., Fazlzadeh, M., Mohammadi-Moghadam, F., Nourmoradi, H., Goudarzi, B., Dindarloo, K., 2018. Preparation, characterization and Cr(VI) adsorption evaluation of NaOH-activated carbon produced from Date Press Cake; an agro-industrial waste. Bioresource Technol. 258, 48–56. https://doi.org/10.1016/j.biortech.2018.02.106.